# CITY OF WOODWAY # ANNUAL WATER QUALITY REPORT 2025

# Consumer Confidence Report Contact: Mitch Davison, Community Services & Development Director Email: mdavison@woodwaytexas.gov Telephone: 254.772.4050 Website: www.woodwaytexas.gov.

DRINKING WATER

## You may be more vulnerable than the general population to certain microbial contaminants, such as cryptosporidium, in drinking water. Infants, some elderly or immune-compromised people such as those undergoing chemotherapy for cancer, those who have undergone organ transplants; those who are undergoing treatment with steroids; and people with HIV/AIDS or other immune system disorders can be particularly at risk from infections. You should seek advice about drinking water from your physician or health care provider. Additional guidelines on appropriate means to lessen the risk of infection by cryptosporidium are available from the SAFE DRINKING WATER HOTLINE at (800) 426-4791.

## Public Participation Opportunities

## The City of Woodway Water Utility Department is governed by the Woodway City Council. The City Council meets on the 2nd and 4th Mondays each month at 5:30 p.m. at the Woodway City Hall located at 922 Estates Drive. To learn more about future public meetings (regarding drinking water) or to request to schedule one, please contact us at (254) 772-4050 or (254) 772-4480.

## Our Drinking Water is Regulated

## This report is summary of the quality of the water we provide our customers. The analysis was made by using data from the most recent U.S. Environmental Protection (EPA) required tests. We hope this information helps you become more knowledgeable about your drinking water.

## En español

## Este informe incluye información importante sobre el agua potable. Si tiene preguntas o comentarios sobre este informe en español, favor de llamar al tel. (254) 772-4050 para hablar con una persona bilingüe en español.

Prevent Storm Water Pollution

Motor oil, paint, fertilizer, or anything that is on the ground when it rains all gets washed into the storm drain system along with the rain. Unlike wastewater (which is treated), storm water runoff goes into creeks, lakes, and rivers. This is why disposing of oil, pesticides, and other chemicals properly is very important. Always use and dispose of chemicals in accordance with product labels.

WATER SOURCES

## Source of Drinking Water

## The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells. As water travels over the surface of the ground, it dissolves naturally occurring minerals, and in some cases, radioactive materials, and can pick up substances resulting from presence of animals or from human activity. Contaminants that may be present in source water before treatment include microbial contaminants, such as viruses & bacteria, which may come from sewage treatment plants, septic systems, and agricultural livestock operations & wildlife. Inorganic contaminants, such as salts and metals, which can be naturally occurring or results from urban storm water runoff, industrial, or domestic wastewater discharges, oil, and gas production, mining or farming. Pesticides and herbicides, which may come from a variety of sources such as agriculture, urban storm water runoff and septic systems. Radioactive contaminants, which can be naturally occurring or be the result of oil and gas production and mining activities.

## Reduce the F.O.G. (fats, oils & grease)

## Fats, oils, and grease that are poured into drains or toilets solidify as they cool in the pipes, clogging up the system. You can help by practicing these FOG reducing tips:

DO NOT: put food down the drain; use the disposal excessively; pour oil or grease down the drain; or rinse grease from cookware into the sink or drain.

DO: cover sink drain with catch baskets and empty into waste bin; dry-wipe oil/grease from cookware; put used cooking oil in a covered container and dispose of.

Where do we get our drinking water?

Our drinking water is obtained from surface and ground water sources. It comes from six (6) wells located within the City, pumping from the Hosston Member of the Trinity Group Aquifer with supplemental supply from Waco. Assessment for your drinking water source(s) is currently being updated by the Texas Commission on Environmental Quality (TCEQ). This information describes the susceptibility and types of constituents that may come into contact with your drinking water source based on human activities and natural conditions. The information contained in the assessment allows us to focus on our source water protection strategies. Some of this source water assessment information will be available later this year on Texas Drinking Water Viewer at http://dwv.tceq.texas.gov . For more information on source water assessment and protection efforts on our system, contact Woodway Community Services at (254) 772-4050.

Did you know?

The human body is about 75% water, and by the time a person feels thirsty, his or her body has lost over 1% of its total water amount. Although soft drinks, coffee, and tea are made up almost entirely of water, they also contain caffeine, which can prevent water from traveling to necessary locations in the body. A person can survive about a month without food, but only 5 to 7 days without water. Find more interesting water facts at: allaboutwater.org.

## ALL drinking water may contain contaminants
When drinking water meets federal standards there may not be any health-based benefits to purchasing bottled water or point of use devices. Drinking water, including bottled water, may reasonably be expected to contain at least some small amounts of contaminants. The presence of contaminants does not necessarily indicate that water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the EPA’s Safe Drinking Water Hotline (1-800-426-4791).

## Secondary Constituents

## Many constituents (such as calcium, sodium, or iron) which are found in drinking water, can cause taste, color, and odor problems. The taste and odor constituents (secondary constituents) are regulated by the State of Texas. These constituents are neither cause for health concern nor requirement to be reported in this document; however, they may greatly affect the appearance and taste of your water.

Definitions

Maximum Contaminant Level (MCL): The highest permissible level of contaminant in drinking water. MCLs are set as close as to the MCLGs as feasible using the best available treatment technology.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected health risk. MCLGs allow for a margin of safety.

Maximum Residual Disinfectant Level (MRDL): The highest level of disinfectant is allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

Treatment Technique (TT): The required process is intended to reduce the level of contaminants in drinking water.

Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

Abbreviations

| MFL | million fibers per liter (a measure of asbestos) |
| --- | --- |
| NTU | nephelometric turbidity units (a measure of turbidity) |
| pCi/L | picocuries per liter (a measure of radioactivity) |
| ppb: | micrograms per liter or parts per billion - or one ounce in 7,350,000 gallons of water. |
| ppm: | milligrams per liter or parts per million - or one ounce in 7,350 gallons of water. |
| ppt | parts per trillion, or nanograms per liter (ng/L) |

# 2025 Water Quality Test Results

Regulate Contaminants; Collection Date; Highest Value; Range; Unit; MCL; MCLG; Typical Source. Barium; 2/10/2025; 0.0732; 0.0732; ppm; 2; 2; Discharge of drilling wastes; Discharge from metal refineries; Erosion of natural deposits. Cyanide; 3/8/2025; 20; 0-20; ppb; 0; 200; Discharge from steel/metal factories; Discharge from plastic and fertilizer factories.. Dibromochloromethane; 7/24/2025; 10.2; 1.5-10.2; UG/L; 0; 0.06. Fluoride; 4/24/2025; 1.78; 0.72-1.78; ppm; 4; 4; Erosion of natural deposits; Water additive which promotes strong teeth. Discharge from fertilizer and aluminum factories.. Nitrate; 2/10/2025; 0.26; 0-0.26; ppm; 10; 10; Runoff from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits.. Nitrate-Nitrite; 4/5/2025; 0.25; 0.09-0.25; ppm; 10; 10; Runoff from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits.. Selenium; 2/10/2025; 3.2; 3.2; ppb; 50; 50; Discharge from petroleum and metal refineries; erosion of natural deposits; discharge from mines.. ## Maximum Residual Disinfectant Level Year; Disinfectant; Minimum Level; Maximum Level; MRDL; MRDLG; Unit of Measure; Violation; Source of Chemical. 2025; Chloramine Residual; 0.2; 3.9; 4; 4; ppm; N; Disinfectant used (for Woodway's purchased source water) to control microbes.

Disinfection Byproducts

| Disinfectant Byproducts | Sample Point | Period | Highest LRRAA | Range | Unit | MCL | MCLG | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Halo acetic Acids (HAA5) | 105 Westridge, Woodway | 2025 | 14 | 7.3-17.6 | ppb | 60 | 0 | By-product of drinking water disinfection |
| Total Halacetic Acids (HAA5) | 703 Poage, Woodway | 2025 | 13 | 7-16.5 | ppb | 60 | 0 | By-product of drinking water disinfection |
| TTHM | 105 Westridge, Woodway | 2025 | 30 | 16.8-39.8 | ppb | 80 | 0 | By-product of drinking water chlorination |
| TTHM | 703 Poage, Woodway | 2025 | 27 | 15.8-34.5 | ppb | 80 | 0 | By-product of drinking water chlorination |

Volatile Organic Contaminants Year; Contaminant; Highest Level Detected; Range of Individual Samples; MCLG; MCL; Unit of Measure; Violation; Source of Contaminant. 2024-2025; Ethylbenzene; 0; 0; 700; 700; ppb; N; Discharge from petroleum refineries.. 2024-2025; Xylenes; 0; 0; 10; 10; ppm; N; Discharge from petroleum factories; discharge from chemical factories.. Radioactive Contaminants Radiological Contaminants; Collection Date; Highest Value; Range; Unit; MCL; MCLG; Typical Source. Combined Radium (-226 & -228); 2/15/2025; 1.5; 1.38-1.5; PCI/L; 5; 0; Erosion of natural deposits. Radium-226; 4/13/2025; 1.38; 1.38; PCI/L; 5; 0; Erosion of natural deposits. Radium-228; 7/24/2025; 1.05; 1.05; PCI/L; 5; 0; Erosion of natural deposits. 2025 Fecal Coliform: REPORTED MONTHLY TESTS FOUND NO FECAL COLIFORM BACTERIA 2025 Total Coliform: REPORTED MONTHLY TESTS FOUND NO COLIFORM BACTERIA. Unregulated Contaminants (These contaminants are disinfection byproducts. There is no maximum contaminant level for these chemicals at the entry point of distribution.) Year; Contaminant; Minimum Level; Maximum Level; Unit of Measure; Source of Contaminant. 2023-2025; Bromoform; 1.5; 3.41; UC/L; Byproduct of drinking water disinfection.. 2023-2025; Bromodichloromethane; <1.0; 9.74; UC/L. 2023-2025; Chloroform; <1.0; 7.15; UC/L. 2023-2025; Dibromochloromethane; 1; 9.45; UC/L. Synthetic Organic Contaminants Including Pesticides and Herbicides Year; Contaminant; Highest Level Detected; Range of Individual Samples; MCLG; MCL; Unit of Measure; Violation; Source of Contaminant. 2023-2025; Atrazine; <0.1; <0.1; 3; 3; UG/L; N; Runoff from herbicide used on row crops. 2023; Dalapon; <1; <1; 200; 200; UG/L; N; Discharge from petroleum factories; discharge from chemical factories.. Secondary and Other Constituents Not Regulated (No associated adverse health effects) Year or Range; Constituent; Average Level; Minimum Level; Maximum Level; Secondary Limit; Unit of Measure; Source of Contaminant. 2023-2025; Aluminum; 0.027; 0; 0.053; N/A; Ppm; Abundant naturally occurring element.. 2023; Chloride; 63; 32; 77; N/A; MG/L; Abundant naturally occurring element; used in water purification; byproduct of oil field activity.. 2023-2025; Iron; 0.01; 0; 0.014; N/A; MG/L; Erosion of natural deposits; iron or steel water delivery equipment or facilities.. 2023-2025; Magnesium; 2.65; 1.04; 3.94; N/A; MG/L; Abundant naturally occurring element.. 2023-2025; Manganese; 0.002; 0; 0.57; N/A; MG/L; Abundant naturally occurring element.. 2023-2025; Sodium; 185.93; 64.9; 204; N/A; MG/L; Erosion of natural deposits; by products of oil field activity.. 2023; Sulfate; 109; 40; 112; 300; MG/L; Naturally occurring; common industrial byproduct; byproduct of oil field activity.. 2023; Total Alkalinity as CaCO3; 265; 150; 350; N/A; MG/L; Naturally, soluble mineral salts.. 2023; Total Dissolved Solids; 584; 314; 675; 1000; MG/L; Total dissolved mineral constituents in water.. 2023-2025; Total Hardness as CaCO3; 82; 11.5; 94.4; N/A; MG/L; Naturally occurring calcium.. 2023-2025; Zinc; 0; 0; <0.005; 5; MG/L; Moderately abundant naturally occurring element; used in the metal industry.. Lead and Copper Lead and copper; Period; 90th percentile: 90% of your water utility levels were less than; Range of sampled results (low-high); Unit; AL; Sites over AL; Typical Source. Copper, Free; 2023-2025; 0.1551; 0-0.7929; ppm; 1.3; 1.3; Corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives. Lead; 2023-2025; 0; 0-49.3; ppm; 15; 15; Corrosion of household plumbing systems; erosion of natural deposits.. Lead Service Line Inventory Statement In accordance with EPA regulations, the City of Woodway has completed a comprehensive Service Line Inventory. The inventory confirms that there are no known lead service lines, no galvanized lines requiring replacement, and no unknown service lines in the water distribution system. The Service Line Inventory is available for public review at Community Services & Development, located at 924 Estates Drive, Woodway, Texas 76712, during regular business hours. If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. This water supply is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at http://www.epa.gov/safewater/lead ## Violation During the period covered by this report we had the noted violations. Violation Period; Analyte; Violation Type; Violation Explanation. 1/1/2025-12/31/2025; CONSUMER CONFIDENCE RULE; Monitoring, routine major; No monitoring samples were taken or reported. 12/30/2025-3/3/2026; LEAD & COPPER RULE; Lead consumer notice (LCR); Failed to meet content, delivery, and/or reporting requirements for lead consumer notification. Some people who drink water containing cyanide well more than the MCL over many years could experience nerve damage or problems with their thyroid. Infants and children are typically more vulnerable to lead in drinking water than the general population. It is possible that lead levels at your home may be higher than at other homes in the community because of materials used in your home’s plumbing. If you are concerned about elevated lead levels in your home’s water, you may wish to have your water tested and flush your tap for 30 seconds to 2 minutes before using tap water. Additional information is available from the Safe Drinking Water Hotline (800-426-4761). Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found during these assessments. There are no additional required health effects violation notices. City of Waco (Reported test results) Contaminant; Unit; Highest Level Detected; Min-Max Levels; Maximum Allowable Contaminant Level. Arsenic; ppm; <0.0020; <0.0020-<0.0020; 10. Atrazine; ppb; <0.1; <0.1-<0.1; 3. Barium; ppb; 0.0408; 0.0408-0.0403; 2. Fluoride; ppm; 0.18; 0.17; 4. Turbidity; NTU; 100% of the readings were at or below 0.3 NTU; PASS. 2025 UCMR 5 Sampling Test Results The EPA’s Unregulated Contaminant Monitoring Rule 5 (UCMR5) requires public water systems to test for emerging contaminants, including 29 types of PFAS—man-made chemicals found in consumer products such as food packaging, nonstick cookware, water-resistant clothing, and firefighting foam. These substances are persistent in the environment and difficult to remove from drinking water. PFAS can enter water supplies through contact with runoff, spills, wastewater discharge, or landfills. The EPA is continually studying ways to detect, monitor, and reduce PFAS in drinking water.

| Year | Constituent | Average Level | Minimum Level | Maximum Level |
| --- | --- | --- | --- | --- |
| 2025 | Lithium | 35.3 | 29.9 | 42 |
| 2025 | PFBA | <0.005 | <0.005 | <0.005 |
| 2025 | PFHpA | <0.003 | <0.003 | <0.003 |
| 2025 | PFHxS | <0.003 | <0.003 | <0.003 |
| 2025 | PFNA | <0.004 | <0.004 | <0.004 |
| 2025 | PFOS | <0.004 | <0.004 | <0.004 |
| 2025 | PFOA | <0.004 | <0.004 | <0.004 |
| 2025 | PFDA | <0.003 | <0.003 | <0.003 |
| 2025 | PFDoA | <0.003 | <0.003 | <0.003 |
| 2025 | PFHxA | <0.003 | <0.003 | <0.003 |
| 2025 | PFUnA | <0.002 | <0.002 | <0.002 |
| 2025 | 11Cl-PF3OUdS | <0.005 | <0.005 | <0.005 |
| 2025 | 9Cl-PF3ONS | <0.002 | <0.002 | <0.002 |
| 2025 | ADONA | <0.003 | <0.003 | <0.003 |
| 2025 | HFPO-DA | <0.005 | <0.005 | <0.005 |
| 2025 | PFBS | <0.003 | <0.003 | <0.003 |
| 2025 | 6:2 FTS | <0.005 | <0.005 | <0.005 |
| 2025 | 4:2 FTS | <0.003 | <0.003 | <0.003 |
| 2025 | 8:2 FTS | <0.005 | <0.005 | <0.005 |
| 2025 | PFMPA | <0.004 | <0.004 | <0.004 |
| 2025 | PFPeA | <0.003 | <0.003 | <0.003 |
| 2025 | PFMBA | <0.003 | <0.003 | <0.003 |
| 2025 | PFEESA | <0.003 | <0.003 | <0.003 |
| 2025 | NFDHA | <0.02 | <0.02 | <0.02 |
| 2025 | PFPeS | <0.004 | <0.004 | <0.004 |
| 2025 | PFHpS | <0.003 | <0.003 | <0.003 |
| 2025 | PFTA | <0.008 | <0.008 | <0.008 |
| 2025 | PFTrDA | <0.007 | <0.007 | <0.007 |
| 2025 | NEtFOSAA | <0.005 | <0.005 | <0.005 |
| 2025 | NMeFOSAA | <0.006 | <0.006 | <0.006 |